V SEMESTER B.TECH. IT & CCE

End-Semester Examination, November 2022

SUBJECT: Network Programming and Simulation [ICT 3165]

REVISED CREDIT SYSTEM

(8/11/2022)

Time: 2:00 Hour						                           MAX. MARKS: 40

Instructions to Candidates:

- Answer ALL the questions.

- Missing data, if any may be suitably assumed.

0

| 1. | Configure the network with required commands as shown in Figure 1. (5) - Configure the yellow region with OSPF protocol, white region with RIP protocol and show the routing table of router 2. (8) - Test the connectivity between Laptop 0 and Server0 by using command line. (3) - Configure the VoIP for the below given topology and call IP Phone1 from IP Phone0. (3) - Changes the Hello Interval timer to 20 seconds of router 2. (1) Note: - While configuring the network use same IP address as shown in figure 1. Figure 1: Network Configuration | 20 |
| --- | --- | --- |
| 2 | Write two separate C programs (one for server and the other for client) using socket APIs, to implement the following connection-oriented client-server model to create a book database at the server side (structure) and store the following information: Book_ID,Book_Name, Title, Publisher_Name, Genre. Save the information for minimum 3 books. Note that the results must be displayed at the client side. (5) The client program should display a menu for the following operations. - Insert (5) - Update (5) - Search (4) - Exit (1) Client and Server will perform following operations as per menu options: - Insert the information of each book and Display. - Update the Genre of the book titled “Harry Potter” from Comic to Fantasy. - Search and display the name of the author with maximum number of publications. - Exit the application with “Bye” message. | 20 |

| Q. No |  | Comments (Full / Partial (Min/Max) / No output) | Allotted Marks | Obtained Marks |
| --- | --- | --- | --- | --- |
| 1. |  |  | 5 |  |
|  | a. |  | 8 |  |
|  | b. |  | 3 |  |
|  | c. |  | 3 |  |
|  | d. |  | 1 |  |
| 2. |  |  | 5 |  |
|  | a. |  | 5 |  |
|  | b. |  | 5 |  |
|  | c. |  | 4 |  |
|  | d. |  | 1 |  |